ASIAN DRAGON’S SUN POWER

1) Catalog/technical data of the heat pipe solar heater with ss inner and outer tank,

2) Design diagram / layout of the solar tank.

3) Design diagram / layout of the bracket. Please take into account that for Colombia the Inclination angle must be 15 or 20

Specification of Dragon’s SPP

Inner tank: diameter 360mm, 1.2mm thickness SUS304-2B stainless steel

Outer tank: diameter 470mm, 0.4mm thickness SUS201-BA stainless steel

Insulation: polyurethane 55mm

Frame: Stainless steel 1.2mm

Angle: 20 degrees

Working pressure: 0.6MPa    Testing pressure: 0.9MPa

Central distance of holes: 80mm

Thermal conductivity: 0.022w/m.k

Sealing rubber: 110 methyl silicone rubber with vinyl

Dustproof rubber: EPDM

Tailstock: Nylon

Daily efficiency: > 50%

Heat loss coefficient: 10W/㎡

Electric booster: Optional 1500-2500W

Reflector is available

Inner tank adopts 1.2~2.0mm SUS3042B stainless steel and automatic wending technology, which decrease the corrosion and against the 0.6Mpa pressure. Outer tank use 0.4mm 201 BA stainless steel or color steel. Insulation adopts polyurethane which has good insulation efficiency, thickness is 55mm.

●The tubes and supports are adjustable, power bearing is average and assure the tubes to use long time.

●Three-layer(ALN/ALN-SS-CU) concentric borosilicate glass tubes

1.6mm thickness.

Φ58 ×1,800mm with heat pipe, resistance cold -35

Absorb rate≥96%

Thermal radiation rate≤4%.

Resist25mm hail, using life lasts 15years.

● It has advantages of no water in tube, no filthy, no broken tube which is antifreeze, unlimited time of the inputting water and pressure operation

Inner tank

Design