ADIEQ5090F 4X4 off road wheeled armored anti-riot vehicle

Payment terms: T/T,down payment 60%,the 40% balance cash before delivery.

Loading port: any port of China to XXX

FOB: US$226,000.00/Unit RHD

ADIEQ5090F 4X4 off road wheeled armored anti-riot vehicle

| Model |  | 4X4 off road wheeled amored anti-riot vehicle |
| --- | --- | --- |
| Overall dimension L*W*H |  | 6570x2490x3050 mm |
| Wheelbase(mm) |  | 3200 |
| Tread (mm) |  | 1970 |
| Approach angle(°) |  | 40 |
| Departure angle(°) |  | 35 |
| Min. ground clearence(mm) |  | 300 |
| Curb weight(kg) |  | 4500 |
| GVW (kg) |  | 9000 |
| Max. speed(km/h) |  | 120 |
| Max. climbing grade(%) |  | more than 60% |
| Max. driving side slope(%) |  | 40% |
| Portrait transit angle(°) |  | 32 |
| Min. swerve diameter(m) |  | no more than 15 |
| Engine | Model | Cummins ISDe180-30 180hp |
|  | Emission standard | Euro III |
|  | Engine type | Water-cooled six-cylinder,4 stroke turbocharged and intercooled diesel |
|  | Displacement | 4.5L |
|  | Max. output | 136kw/2500r/min |
|  | Max. torque | 650N.m/1200~1700r/min |
| Clutch |  | Single, dry disc, φ395 spring,hydraulic booster, far-distance control |
| Transmission |  | Manual, six D drive with Synchronizer, one R drive |
| Divided-moved Ware |  | manual, two position,higher gear ratio1.08,lower gear ratio 1.35 |
| Drive bridge |  | Split Type cast-iron Axle Housing, steel tube semi-axle bushing,universal-joint cross couping with front/rear axle, Differential Lock on front/rear axle |
| Frame |  | Run-in frame with two slot side carling |
| Suspension |  | Independent suspension of Front/Rear axle |
| Brake system |  | Driving arrester: four wheel double-loop air brake, drum arrester, two brake shoes in front, double-loop pump in series, air-pressure control, two air bag brake in front and rear, front dual disc and Parking arrestor rear wheel brake, drum arrester, air-pressure control |
| Steering system |  | Front wheel turning, gear wheel and hydraulic power assisted steering system |
| Tyre |  | 295/60R22.5 Meridian tyre without inner tube |
| Protection performance |  | Vehicle Resistance Level: Four sides of this vehicle can resist 56 type submachine gun, 7.62mm steel cored warhead shooting. The top and bottom of vehicle can resist 56 type submachine gun with steel cored warhead shooting. The whole vehicle can resist demolition by explosion device on the side and bottom. |
| Driving type |  | 4x4 off road Left hand driving or right hand driving |
| Air conditioner |  | option |

Note: price is not include Weapon .