ADIEQ2050F 4X4 off road armored patrol vehicle
Payment terms: T/T

Loading port: any port of China

FOB: US$222,000.00/Unit

4X4 off road patrol vehicle

| Model |  | 4X4 off road patrol vehicle(APC) |
| --- | --- | --- |
| Overall dimension (mm) |  | 4970/5900x2134/2400x1960/2000 |
| Wheelbase(mm) |  | 3300-4000 |
| Wheel Tread front/rear (mm) |  | 1820/1820 |
| Approach angle(°) |  | 47 |
| Departure angle(°) |  | 40 |
| Min. ground clearence(mm) |  | 406 |
| Curb weight(kg) |  | 3250 |
| GVW (kg) |  | 7750 |
| Max. speed(km/h) |  | 134 |
| Max. climbing grade(%) |  | more than 60% |
| Max. driving side slope(%) |  | 40% |
| Portrait transit angle(°) |  | 32 |
| Min. swerve diameter(m) |  | no more than 15 |
| Tire specification |  | 37x12.5R16.5 |
| Engine | Model | Cummins ISDe180-30 180hp |
|  | Emission standard | Euro III |
|  | Engine type | Water-cooled six-cylinder,4 stroke turbocharged and intercooled diesel |
|  | Displacement | 4.5L |
|  | Max. output | 136kw/2500r/min |
|  | Max. torque | 650N.m/1200~1700r/min |
| Clutch |  | Single, dry disc, φ395 spring,hydraulic booster, far-distance control |
| Transmission |  | Manual, six D drive with Synchronizer, one R drive |
| Divided-moved Ware |  | manual, two position,higher gear ratio1.08,lower gear ratio 1.35 |
| Drive axle |  | Split Type cast-iron Axle Housing, steel tube semi-axle bushing,universal-joint cross couping with front/rear axle. |
| Frame |  | Run-in frame with two slot side carling |
| Suspension |  | Independent suspension of Front/Rear axle |
| Brake system |  | Driving arrester: four wheel double-loop air brake, drum arrester, two brake shoes in front, double-loop pump in series, air-pressure control, two air bag brake in front and rear, front dual disc and Parking arrestor rear wheel brake, drum arrester, air-pressure control |
| Steering system |  | Front wheel turning, gear wheel and hydraulic power assisted steering system |
| The trench width (mm) |  | 850 |
| Wading depth (mm) |  | 760 |
| Vertical obstacle height (mm) |  | 440 |
| Protection performance |  | 100 meters from the front windshield glass can withstand 7.62 mm rifle design, the body can withstand 7.62 mm armor-piercing shot. 100 meters from the side of the body can fully withstand 7.62 mm rifle shooting, which can effectively protect explosive shock wave. After the installation of V underbody armor protection to enhance the level of protection type. |
| Vehicle function |  | Fully meet the forces of the siege of the border areas, low-intensity conflict in the border plateau, rapid deployment, within the scope of the technical and tactical indicators city theater of battle. |
| Driving type |  | 4x4 off road Left hand driving |
| Air conditioner |  | With A/C |
| Vehicle |  | Color can be customized according to customer requirements |

Note: Photo for reference only,subject to manufacture’s upgrades