Steyr 6x6 off road special vehicle

Payment terms: T/T,down payment 40% arrange for manufacturing,the 60% cash before delivery(or 70% balance L/C at sight can be accepted)

Delivey time: depends on customer order quantity.

Loading port/destination port: Any port of China to XXXX
FOB: US$47,760.00/Unit

Steyr 6x6 off road special vehicle specification

| Steyr 6x6 off road special vehicle (long cargo box) |  |  |  |
| --- | --- | --- | --- |
| Driving type |  | 6x6 off road for military |  |
| Overall Dimension (L*W*H) mm |  | 9915x2480x3350 |  |
| Cargo box Dimension (L*W*H) mm |  | 7220x2368x620(long cargo box) |  |
| Wheelbase (mm) |  | 4325+1350 |  |
| Curb weight (kg) |  | 11650 |  |
| Front overhang/Rear overhang (mm) |  | 1576/3030 |  |
| Front /Rear track (mm) |  | 1939/1800 |  |
| Approach angle/Departure angle |  | 21/17 |  |
| Max speed (km/h) |  | 80 |  |
| Cab | Hydraulic shock absorb adjustable seat, adjustable steering. Safety belt, cabin front protection, cooler protection, light protection, upward exhaust pipe. |  |  |
| Engine | Model |  | WD615.50 WEICHAI DIESEL ENGINE |
|  | Feature |  | MAN EVB tech. Diesel, 4-stroke direct injection, 6 cylinder in line, radiator, turbo-charging. |
|  | Bore /stroke (mm) |  | 126x130 |
|  | Displacement (L) |  | 9.726 |
|  | Power |  | 290hp/213kw |
|  | Rated speed (r/min) |  | 2000 |
|  | Torque (N.m) |  | 1160 |
|  | Emission |  | EuroII |
| transmission | Model |  | FAST gearbox |
|  | Shift mode |  | Manual 9 forward 1 reverse |
|  | Torque (N.m) |  | 1350 |
|  | First shift ratio |  | 7.285 |
| Electrical | Battery |  | 12V/135Ah |
|  | Generator |  | 28V-35A |
|  | Starter |  | 5.4Kw/24V |
| Spring leaf | Type |  | Spring leaf |
|  | Front |  | 9 |
|  | Rear |  | 9+6 |
| Axle load weight(T) | Front |  | 6.5t Steering axle |
|  | Rear |  | 13t Double reduction, hub redactor driving |
| Steering | Left hand drive Power steering |  |  |
| Tire | 14.00-20 military tyre 6+1 spare |  |  |
| Clutch | 430 Diaphragm type, single dry frictional disc, hydralic boosting |  |  |
| Fuel tank | 380L steel tank |  |  |
| Air conditioner |  |  | With A/C |