Shiyan Qianfeng Industry & Trade Co.,Ltd

Add:No.208#,Autoparts Plaza,Zhenya Area,Bailang Middle Road,Shiyan ,Hubei ,China

Tel:0086-719-8318338   Fax: 0086-719-8365165   E-mail:charley222@126.com

Dongfeng EQ5160G 6x6 off road military special truck
Payment terms: T/T,down payment 30% arrange for manufacturing,the 70% cash before delivery.

Loading port/destination port: Any port of China to
FOB: US$47,100.00/Unit

Dongfeng EQ5160G 6x6 off road military special truck specification

| Brand | Dongfeng | Drive type | 6x6 latest new design militery truck |
| --- | --- | --- | --- |
| Product name | Off-road military cargo truck | G.V.W (kg) | 16050 |
| Curb weight (kg) | 7955 | Overall dimension L*W*H(mm) | 9630*2470/2480*3350 9915*2470*3350 |
| Load weight (kg) | 8095 | Cargo Box Dimension L*W*H(mm) | 7200*2300*620 7200*2368*620 |
| Wheel base (mm) | 4350+1350 4650+1350 | F/R axle load weight(kg) | 6.5t Steering axle /13t Double reduction, hub redactor driving |
| Quantity of leaf spring | 9/12 | Quantity of tires | 6 + 1 spare |
| Tyre type | 14.00-20 12.5R20 | Approach/ Departure angle | 35/34° |
| F/R suspension (mm) | 1110/1585 | Front track (mm) | 1876 |
| Rear track (mm) | 1870,1870 | Max.vehicle speed | 90km/h |
| Passengers in cab | 3 | Chassis model | EQ2102GJ |
| Engine | Model | WD615.50 WEICHAI DIESEL ENGINE |  |
|  | feature | MAN EVB tech. Diesel, 4-stroke direct injection, 6 cylinder in line, radiator, turbo-charging. |  |
|  | Bore /stroke (mm) | 126x130 |  |
|  | Displacement (L) | 9.726 |  |
|  | Power | 290hp/213kw |  |
|  | Rated speed (r/min) | 2000 |  |
|  | Torque (N.m) | 1160 |  |
|  | Emission level | Euro 2 |  |
| Gearbox | Model | FAST gearbox |  |
|  | Shift mode | Manual 9 forward 1 reverse |  |
|  | Torque (N.m) | 1350 |  |
|  | First shift ratio | 7.285 |  |
| Electrical | Battery | 12V/135Ah |  |
|  | Generator | 28V-35A |  |
|  | Starter | 5.4Kw/24V |  |
| Fuel tank | 380L steel tank |  |  |
| Vehicle color | According to customer request but need to inform us in advance |  |  |
| Brake system | Service brake | Double circuit pneumatic (air) brake |  |
|  | Parking brake | Spring-Power drum brake |  |
| Cabin | Description | One and half row, standard configuration,power steering, with Air conditioner |  |
| Performance | Max.grade ability | 60% |  |
|  | Braking distance | 8m |  |
|  | Minimum turning diameter | 18.8m |  |

Company name: Shiyan Qianfeng Industry & Trade Co.,Ltd
Add:No.208#,Zhenya Area, Autoparts Plaza,Bailang Middle Road,Shiyan,Hubei,P.R.China
E-mail: charley222@126.com
Tel:0086-719-8318338
Fax: 0086-719-8365165
Mobile/Whatsapp: 0086-13972506691
Skype: Charlie3378
Http//：www.qftruck.en.alibaba.com

Note: Photo for reference only,subject to manufacture’s real object