EQ5162N Dongfeng 6x6 ambulance truck

Payment terms: T/T

Loading port/destination port: any port of China to

FOB: US$46,880.00/Unit

| the new vehicle technical parameters |  |
| --- | --- |
| vehicle name | ADIEQ5162N Off-road truck ambulance（double cabin） |
| （mm） overall dimension L*W*H | 7600 * 2550 * 3550 |
| fuel sort | diesel |
| emission standard | Euro III |
| displacement（ml）/power（kw）displacement | 5900/ 140 |
| steering type | steering wheel |
| （mm） box inner size L*W*H | 4800 * 2250 * 2480 |
| axle | 3 |
| wheelbase | 3500+1250 |
| spring leaf | 10/12 |
| wheel specification | 12.5R20 |
| wheel numeral | 6 |
| （mm）wheel span front/rear | 1876 / 1870 |
| （kg）max gross weight | 16050,11650 |
| （kg）specified load weight | 7900,3500 |
| （kg）curb weight | 7955 |
| cab passengers | 3 |
| approach/departure angle | 35/34 |
| （kg）front/rear suspension | 1385/1515 |
| （km/h）top speed | 90 |
| （kg）：axle load | 4020/12030(double axle),3410/8240 |
| chassis model/manufacturer | EQ2162GJ Dongfeng vehicle co |
| ：engine model/manufacturer | EQB190-33 Dongfeng Cummins diesel engine co.,ltd |

2、医medical cabin arrangement schematic diagram

3、医疗舱布置平面示意图:medical cabin plane diagram

Modification Items

| 1 | Medical cabin air conditioning (Originalair-conditioning system ) |
| --- | --- |
| 2 | Medical cabin with ventilation system installed |
| 3 | Medical cabin with heater system installed |
| 4 | Field ambulance with lights installed on the left and right sides of roof and rear roof |
| 5 | Medical cabin fluorescent lighting |
| 6 | Three rotatable searchlights |
| 7 | one UV disinfection ultraviolet light |
| 8 | Intelligent power management system, equipped with electricity equipment battery, 220 V power inverter, charger |
| 9 | 10 m mobile cables, connecting external power supply |
| 10 | Right side of the vehicle body with 220V/15A waterproof, protective cover, with an external power supply socket installed |
| 11 | two 220V socket in the medical cabin |
| 12 | two 12V socket in the medical cabin |
| 13 | Driving cab with alarm lamp and alarm lamp controller |
| 14 | double-row Superintendent blue flashing lights installed on the front of the roof |
| 15 | 100w vehicle loudspeakers inside the superintendent flashing lights |
| 16 | square Superintendent blue Strobe installed on the left and right sides of the outer roof and rear roof |
| 17 | Pillar Superintendent blue Strobe installed on the tail of the roof |
| 18 | Complete separated Separation wall installed between the driving cab and medical cabin, sliding window installed on the separation wall. |
| 19 | install equipment counter on which can lay defibrillator, ventilator, aspirator inside the left side of the medical cabin.(or designed to counter Block + single small equipment cabinet according to customer requirements) |
| 20 | On the right side in the medical cabin,install Long rows of seats with three seats, fitted with the two-point seat belts in a drawer, under the seat can be placed spade stretcher and debris |
| 21 | two 10 litres’ oxygen cylinders |
| 22 | Oxygen valve, supply of oxygen, attract private-interface connector (one oxygen interface, one air interface ) |
| 23 | Medical bulkheads with corrosion resistance, impact resistance, flame retardant, anti-ultraviolet, adopt special engineering materials processing and installation. |
| 24 | Acid and alkali, fire prevention, anti-skid \ anti-static floor of the medical cabin, the four neighboring arch-bound to prevent the accumulation of dust and anti-skid for the rinse water |
| 25 | Install a set of medicine cabinets, instruments counters and other equipment cabinets. |
| 26 | Install Nurses console in the medical cabin |
| 27 | Install trash bin on the left inside the cabin |
| 28 | Folding up suspention device for the infusion bottle and safety handrails intalled on the top of the cabin. |
| 29 | fire extinguisher(1 kg) and stent |
| 30 | On both sides of the medical cabin, window, door, the back door windows were pasted by 2 / 3 special opaque film for car. |
| 31 | Automatic vehicle stretcher |
| 32 | front and rear interphones |
| 34 | Visualization reversing surveillance system |
| 35 | wash basin |