COMMERCIAL OFFER

| No. | Descriptions | Qt | Per Price | Total Amount |
| --- | --- | --- | --- | --- |
| 1 | GE Gas Turbines Model:LM2500Gen04 Emission:Dry Low Emission Original Year:2011 Refurbishment Date:2026 Delivery time:NOV2026 Refurbishment unit:GE USA Overhaul report,TEST report by Ge USA(NOV2026) Output after overhauled:31mw to 32mw Hours:zero after overhauled Fuel:Natural Gas Generator (will be booked depending on buyer requirement) Voltage:10500v (or buyer may choice voltage) Frequency:50Hz( or buyer may choice frequency) Generator(50hz) Manufacturer:Nanqi turbine factory LTD Generator(60hz) Manufacturer:may choice depending on buyer requirement Control:GE WOODWARD Payment : 10% escrow to reserve 20% after signing contract The balance before shipping Delivery time:Nov2026 Delivery location:China or Usa1 | 1 | $38,900,000usd CIF Any Major Port | $38,900,000usd CIF Any Major Port |

- Payment of the Contract Price shall be made as follows: 10% to reserve into escrow 20% TT after signing contract

GAS TURBINE GENERATOR PACKAGE

LM2500+G4

SCOPE OF SUPPLY and TERMS

### 项目：	TBD

PROJECT:

项目地点：	TBD

PROJECT LOCATION：

项目编号：	TBD

PROJECT NO:

#### 编制Prepared by:	日期Date :2025

1.供货范围Scope of Supply

燃气轮机

#### Gas Turbine

LM2500+G4型燃气轮机为双轴燃气轮机，由一个燃气发生器和一台六级低速动力涡轮机构 成。动力输出轴穿过排气总管内的轴隧，通过挠性联轴器将轴端功率传递给热端的从动设 备。燃气发生器由可变几何压缩机、干式低排放燃烧器（DLE）、高压透平、控制装置及 附件构成。动力涡轮机是一个轴转速为3000RPM的六级低压透平，以气动耦合的方式联接 到燃气发生器上，由燃气发生器的排出气体驱动。

The LM2500+G4 Gasturbine is a two-shaft engine consisting of a gas generator and a six stage low speed power turbine. Shaft power is transmitted to the driven equipment at hot end by means of a flexible coupling extending through a shaft tunnel within the exhaust collector. The gas generator consists of a variable geometry compressor, a Dry Low Emission (DLE) combustor, a high pressure turbine, control device and accessories. The power turbine with shaft speed of 3,000 rpm, is a six stage, low pressure turbine, aerodynamically coupled to the gas generator and driven by the gas generator exhaust gas.

发电机

#### Generator

发电机为两级三相无刷励磁形式。发电机能够保证在各种环境温度的条件下承载燃气轮机 全工作范围内的连续输出功率。中性点柜和出线柜安装在发电机的旁边，以供买方的电力 接线。发电机安装在客户准备的混凝土基座上。

The generator is characterized as a three-phase, two-pole brushless exciter type.The generator is capable of handling the full continuous power of the gas turbine at various ambient temperatures throughout the operating ranges. Neutral and line side cubicles are located beside of the generator for the customer’s power connections. The generator is mounted on a concrete pad prepared by customer.

燃机罩壳

#### Turbine Enclosure

成套设备中包括了一个用于防风雨和隔音的燃机罩壳。罩壳采用室外安装设计 。 罩壳有 助于在燃机全负荷运行及稳定运行期间，由燃机（及通风系统）产生的噪音平均值，在距 离设备1米、高出燃机罩壳的基础1.5米处降低到85dB(A)。

The package is supplied with a weatherproof acoustic enclosure for the turbine. The turbine enclosures are designed for outdoor installation. The enclosure (with associated ventilation system in operation) provides average noise emission pressure level 85 dB(A) at 1m distance, measured at 1.5m above package base in a free field condition, during full load and steady operation.

燃机底座

#### Turbine Baseplate

燃机底座为钢制，设计提供足够的强度及现场安装要求。燃机底座根据IBC2009规范设计。

A steel baseplate is designed to provide suitable strength and site installation provisions for the gas turbine. The baseplate is also designed in accordance with IBC2009.

空气进气系统

#### Air Inlet System

燃气轮机机组包括模块化设计的静态空气过滤系统，该系统包括防风雨装置、入口滤网、 粗过滤器和尾段过滤器。过滤后的空气被分成两部分，用于燃机轮机的助燃空气和用于燃 机罩壳的通风空气。

The gas turbine package is supplied with a modular design static air filtration system consisting of weather hood, inlet screens, a pre-filter and a final barrier filter. The filtrated airflow is divided into two streams, combustion air for the gas turbine and ventilation air for the turbine enclosure.

管道连接布置

#### Piping Connections Arrangement

燃气轮机发电机组采用室外布置，侧向排气。排气法兰可以根据用户的需求布置在燃机的 左侧或者右侧。燃气轮机的辅机模块以及买方的管道连接位于燃机的左侧，另一侧要留出 足够的空间作检修用。

The gas turbine generator package is outdoor arrangement with side exhaust. The gas exhaust flange can be left or right side of turbine, according to the requirements of customer. The auxiliary skid and customer piping connections also can be left or right side of gas turbine. Adequate spaces should be reserved on the other hand for turbine maintenance.

气体燃料系统

#### Gas Fuel System

燃气轮机采用了由电控燃料流量控制阀控制的干式低排放天然气燃烧系统。在不使用注水 或注蒸汽的条件下，能够将氮氧化物的排放量降低至50mg/NM3。气体燃料必须符合HDGT 的相关技术规范

The gas turbine is supplied with a Dry Low Emission (DLE) natural gas fuel system using electronically controlled fuel-metering valves. This system reduces NOx emissions to 50 mg/NM3 without water or steam injection. Gas fuel must meet HDGT Fuel Specification.

润滑油系统

#### Lube Oil Systems

成套设备配备了两套独立的润滑油系统，一套用于燃气轮机（合成油），另一套用于发电 机（矿物油）。采用管壳式换热器对润滑油进行冷却，EPC负责提供冷却水。

The package is supplied with two separate lube oil systems, one for the gas turbine (synthetic oil) and one for the generator (mineral oil). Shell Tube Exchangers are used for cooling the lube oil with cooling water to be supplied by EPC.

电动液压启动系统

#### Electro-Hydraulic Start System

燃气轮机配有液压启动系统，包括电动马达驱动的液压泵机组、过滤器、冷却器和控制器， 安装在辅助设备模块上。液压马达安装在燃气轮机的辅助齿轮箱上，以转动燃气发生器的 主轴。

The gas turbine is supplied with a hydraulic starting system, which includes an electric, motor-driven, hydraulic pump assembly, filters, cooler and controls, mounted on the auxiliary equipment module. A hydraulic motor is mounted on the gas turbine accessory gearbox to turn gas generator shaft.

防火系统

#### Fire Protection System

成套设备提供一个工厂组装好的防火系统，包括安装在燃气轮机罩壳内的光学火焰检测器、 碳氢化合物检测器、热检测器、管道和喷嘴。防火系统包括安装于一个单独模块上的二氧 化碳气瓶。还包含一个24V直流电池组和独立的充电器，对防火系统提供电源。所有警报 和停机，在燃机控制柜（TCP）上显示通知。

The package is supplied with a factory installed fire protection system complete with optical flame detection, hydrocarbon sensing and thermal detectors, piping and nozzles in the turbine compartments. The fire protection system includes cylinders containing CO2 mounted on a separate skid. A 24 V DC battery and charger to power the fire protection system are also included. All alarms and shutdowns are annunciated at the turbine control panel (TCP). An alarm sounds at the turbine if the gas detectors detect high gas levels, or if the system is preparing to release the CO2.

数字控制系统

#### Digital Control System

成套设备配备有独立的控制柜，适于安装在由客户提供的室内非危险区域。控制系统配有 Woodward Micronet＋燃料控制器、Rx3i GE IP顺序控制器、振动监视器、防火系统、数 字仪表和发电机数字保护继电器模块及显示重要数据的人机界面（HMI）。报警和停机事 件自动显示在HMI显示器上。提供两个以太网TCP/IP或Modbus端口从而将机组情况（状态、 压力、温度等）传输到客户的DCS系统。配备双联  100% 容量充电器的专用24 V DC电池 系统为控制柜供电。

The package is supplied with a free-standing TCP suitable for mounting in an indoor, non-hazardous area provided by customer. The control system features a Woodward Micronet+ fuel controller and Rx3i GE IP sequencer, a vibration monitor, a fire protection system, a digital meter, a digital generator protective relay module and HMI (human machine interface) display of key discrete and analog data. Alarm and shutdown events are displayed on the HMI automatically. Two Ethernet TCP/IP or Modbus Ports are provided to transmit unit conditions (status, pressures, temperature, etc.) to the customer's distributed control system. Power for the control panel is provided by a dedicated 24 V DC battery system with dual 100% capacity chargers.

在线及离线水洗系统

#### On-line and Off-line Water Wash System

成套设备配备了在线/离线清洗系统，允许客户在满功率运行过程或者在离线停机时清洗燃

气轮机的压气机段。客户需要提供满足HDGT要求的清洗用纯净水和清洗剂，并提供压力

为689－827kPag，过滤精度为5微米（绝对值）的压缩空气。

The package is supplied with an on-line and off-line cleaning system, which allows customer to clean the compressor section of the engine during full power operation or off line without load. Customer is required to provide purified water and detergent per HDGT’s specification and air at 689-827 kPag filtered to 5 microns absolute.

#### Extended Scope Options

扩展范围可选项

#### Installation and Commissioning Service

安装和调试服务

HDGT will provide technical advisory services for the installation and commissioning of the equipment provided under this proposal. Installation technical advisory services will include advising the installation of the gas turbine package by the Owner’s construction contractor. Commissioning technical advisory services will include flushing advisory, checkout, and commissioning advisory of gas turbine mechanical systems, checkout and commissioning advisory of gas turbine electrical systems, and checkout and commissioning of the Gas Turbine Generator control system.

HDGT公司将为所供设备的安装和调试提供技术指导服务。安装技术指导服务包括指导业主

的工程承包商进行燃气涡轮机组安装。调试技术指导服务包括对燃气轮机机械系统的冲洗

指导、校验和调试指导，对燃气轮机电气系统的检验和调试指导，以及对燃气涡轮发电机

控制系统的检验和调试。

2. Equipment and payments 设备及付款方式

One gas turbine generator packages delivered to                        （EXW）, and based on

the scheduled delivery dates in Section 5 of this paper: 一套燃气涡轮发电机组、辅助设备，按 EXWork 工厂交货，以本建议书第 5 部分的计划交 货日期为准：

#### Equipment Description设备型号:	LM2500+G4	DLE	GAS	TURBINE GENERATOR

Equipment Payment Terms 设备的付款方式：

- Down Payment 10%, within 15 (fifteen) days after signature of the Contract Agreement by both Parties, to be paid after receipt of the Seller’s invoice.

首付款10%应在双方签署合同协议后十五（15）天内，在收到卖方发票后支付。

- Equipment Payment 90%, in equal monthly installments, commencing thirty (30) days following signature of the Contract Agreement by both Parties until thirty (30) days before the first scheduled Shipment Date, to be paid after receipt of the Seller’s invoice.

90%的货款按月等额分期付款应从双方签署合同协议后三十（30）天开始计算，直到首个计划装

运日前三十（30）天内、在收到卖方发票后支付。

Above payment based on below conditions:

以上付款基于：

- All Payments shall be made by wire transfer directly to

所有付款买方通过电汇方式直接向	司的银行支付。

- Any and all banking charges will be to the Buyer’s account.

任何及所有银行费用均由买方负担。

- Taxes 税务

The price will above excludes any customs duty, sales tax, VAT or any other similar taxes and duties, which may be associated with the purchase of equipment and/or services.

价格将不包含任何关税、销售税、增值税等相关税费。

- Delivery Terms 交货条款

This Paper is based on EX Works (Incoterms 2020) .

本建议书以EXW工厂交货条款为基础。

- Scheduled delivery date(s) 计划交货日期

which will be defined mutually in the final contract stage.

具体交货时间将在双方最终合同阶段确定。

It is agreed and understood that ship date offered is subject to prior sales.

双方同意且理解，所提议的装运日期以有权先售为条件。

- Warranty 质保期

The warranty offered for this paper is for a period of twelve (12) months following the date of first combustion fuel burn for each Unit, or eighteen (18) months from the Delivery date of the engine of each Unit of the Contract Equipment shipped by the Seller, whichever occurs first.

本建议书中的质保期自每台设备第一次燃料燃烧起12个月或自卖方发运每套合同设备引擎

的交付之日起18个月，二者以先到日期为准。

#### Contract Agreement and General Terms and Conditions of Sale 承包协议及一般

销售条款与条件

The information in this document is provided for reference only. No warranty or representation is given, either expressed or implied, concerning the information in this Paper.

本文件所含信息仅作参考。