某项目5×SGT-300-1S供货范围

Scope of Supply in  Project 5×SGT-300-1S

| 序号 No. | 系统名称 System Name | 主体设备型号规格 Specification of the Equipment | 数量 Qty | 单位 Unit | 分包商 Sub-contrator |
| --- | --- | --- | --- | --- | --- |
| 1 | 燃气轮机撬 Gas Turbine Skid | SGT-300，8MW | 5 | 套 set | Siemens Energy |
| 1.1 | 底盘 Base Module | 撬式碳钢结构 Steel construction Skid | 5 | 套 set | Comelf |
| 1.2 | 燃机核心机 Gas Turbine Core Engine | 含10级压气机、2级透平、6个DLE燃烧器、带点火栓及火焰监测环形燃烧室、入口可调导叶、可倾瓦推力轴承、本体燃料管线、冷却及密封空气系统、气缸、底盘、本体油漆保温等 Including 10-stage compressor, 2-stage turbine, 6 DLE burners, annular combustion chamber with ignition bolt and flame monitoring, adjustable inlet guide vane, tilting pad thrust bearing, body fuel line, cooling and sealing air system, cylinder, chassis, body paint insulation, etc | 5 | 套 set | Siemens Energy |
| 1.3 | 齿轮箱 Gear Box | 含主减速齿轮，辅助齿轮，联轴器，监测仪表等 Including main reduction gear, auxiliary gear, coupling, monitoring instrument, etc | 5 | 套 set | Lufkin Gears |
| 1.4 | 润滑油系统 Lube Oil System | 含主润滑油泵、辅助润滑油泵、事故润滑油泵，油雾分离、润滑油过滤器、加热器、冷却器、温度、液位、压力监测仪表、油箱、管道等 Including main lubricating oil pump, auxiliary lubricating oil pump, accident lubricating oil pump, oil mist separation, lubricating oil filter, heater, cooler, temperature, liquid level, pressure monitoring instrument, oil tank, pipeline, etc | 5 | 套 set | Siemens Energy |
| 1.5 | 启动系统 Start System | 含启动电机与液压马达等 Including starting motor and hydraulic motor, etc | 5 | 套 set | Siemens Energy |
| 1.6 | 燃料气系统 Gas Fuel System | 含流量控制阀、过滤器、快速切断阀、放散阀、监测仪表等 Including flow control valve, filter, quick cut-off valve, release valve, monitoring instrument, etc | 5 | 套 set | Siemens Energy |
| 2 | 发电机 Generator | TFW8-4，8MW 含发电机本体、空水冷却器、无刷励磁、温度及振动监测仪表等 Including generator body, air water cooler, brushless excitation, temperature and vibration monitoring instrument | 5 | 套 set | 湘潭电机 XEMC China |
| 3 | 进气过滤系统 Air Intake System | 含防雨罩、水雾分离器、两级静态过滤（G4+F9）、进气道、消音器、膨胀节、平台过道、监测仪表等 Including rain cover, water mist separator, two-stage static filtration (G4+F9), inlet, silencer, expansion joint, platform aisle, monitoring instrument, etc | 5 | 套 set | 城林环保 CLET China |
| 4 | 罩壳及通风系统 Enclosure and Ventilation System | 含露天布置降噪罩壳，带照明、通风风机、检修平台、挡板阀等 Including open-air noise reduction cover, with lighting, ventilation fan, maintenance platform, baffle valve, etc | 5 | 套 set | 城林环保 CLET China |
| 5 | 排气系统 Exhaust System | 含本体、保温、膨胀节（带保温）、监测仪表等 Including body, insulation, expansion joint (with insulation), monitoring instrument, etc | 5 | 套 set | 城林环保 CLET China |
| 6 | 消防系统 Fire Fighting System | 含温感探头、火焰探头、红外可燃气体探测器、CO2钢瓶、喷嘴、就地钢瓶间、连接管道等 Temperature sensing probe, flame probe, infrared combustible gas detector, CO2 cylinder, nozzle, local cylinder room, connection pipe, etc | 5 | 套 set | 美立马 Minimax China |
| 7 | 移动式水洗小车 Compressor Washing Unit | 含水箱、隔膜泵、低压水管、压力表等 Including water tank, diaphragm pump, low pressure water pipe, pressure gauge, etc | 2 | 套 set | 杭汽 HTC |
| 8 | 燃机控制系统 GT Control System | 含PCS7控制系统、HMI操作员站、盘装式紧急跳机按钮、分布式I/O盘等 Including PCS7 control system, HMI operator station, disk-mounted emergency jump-off button, distributed I/O disk, etc | 5 | 套 set | Siemens Energy |
| 9 | 专用工具 Special Tools | 含安装、调试、维护用工具 Including some installation, commissioning, maintenance tools | 1 | 套 set | Siemens Energy& HTC |
| 10 | 备品备件 spare parts |  | 5 | 套 set | Siemens Energy& HTC |
| 11 | 图纸资料 Drawings | 含出厂合格证、图纸及运维手册 Including factory certificate, drawings and operation and maintenance manual | 5 | 套 set | Siemens Energy& HTC |