# SIEMENS SGT-600 GENERATOR PACKAGES

Prepared for:	Maxum International

Document reference:	25-49

Revision:	01

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

CONTENTS

- INTRODUCTION	2

- SCOPE OF SUPPLY	2

  - GAS TURBINE GENERATOR SET	2

  - GENERATOR AND EXCITATION SYSTEM	3

  - CONTROL SYSTEM	3

  - AUXILIARY SYSTEMS	3

  - BALANCE OF PLANT EǪUIPMENT	3

- INTERFACES	3

  - MECHANICAL INTERFACES	3

  - ELECTRICAL INTERFACES	3

  - INSTRUMENTATION G CONTROL INTERFACES	4

- TECHNICAL PERFORMANCES	4

- PROJECT EXPERIENCE	4

- OVERHAUL SCOPE	4

- COMMERCIAL TERMS	5

- CONCLUSIONS	5

SIEMENS SGT-600 PROPOSAL

# INTRODUCTION

#### We are pleased to present this technical offer for the supply of two (2) Siemens SGT-600 60Hz Open-Cycle Gas Turbine Generator Packages. The SGT-600 is a proven 25 MW class unit offering high availability, dual-fuel flexibility, and low emissions (DLE configuration).

#### We thank you for the opportunity to present this proposal for your consideration and are certain it will meet with your expectations and requirements. Should you have any questions, queries, or concerns relating to this proposal please get in touch with us to discuss these at your convenience.

# SCOPE OF SUPPLY

#### This proposal includes the supply of the following equipment and systems for each of the two (2) SGT-600 generator packages:

| NO | EǪUIPMENT | DESCRIPTION | ǪTY (SETS) |
| --- | --- | --- | --- |
| 1 | Gas Turbine Gen Set | Siemens SGT-600 open-cycle gas turbine (25 MW, 60 Hz, YOM 2008) with Dry Low Emissions combustor system | 2 |
|  | Generator C Excitation System | ABB LBA-1120 LC 4-pole synchronous generator with brushless excitation and AVR | 2 |
| 2 | Control System | Siemens AC160 control panels with PCS7- compatible workstations and vibration monitoring | 2 |
| 3 | Auxiliary Systems | MOOG fuel valves, lube oil system, main gearbox, HVAC, fire protection, gas detection skids | 2 |
| 4 | Balance of Plant Equipment | Fuel skid, starter transformer, DC battery module, MCC panels, foundations and spares | 2 |

## GAS TURBINE GENERATOR SET

#### Siemens SGT-600 open-cycle gas turbine (25 MW, 60 Hz)

#### Year of Manufacturing: 2008

#### Dry Low Emissions (DLE) combustor system

#### Dual-fuel capability (natural gas and liquid fuel)

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

B2 flex coupling gas generator rotor

SIEMENS SGT-600 PROPOSAL

## GENERATOR AND EXCITATION SYSTEM

#### ABB LBA-1120 LC 4-pole synchronous generator (26 250 kVA @ 0.8 pf, 13800 V)

#### Brushless excitation with dual automatic voltage regulator (AVR)

#### Pulse-filtration and air-cooled stator cooling system

## CONTROL SYSTEM

#### Siemens AC160 turbine control panel (PCS7-compatible)

#### Operator and engineering workstations

#### Vibration monitoring and protection modules

#### DC supply and distribution system

## AUXILIARY SYSTEMS

#### MOOG fuel control valves and actuators

#### Frequency drives for lube oil and auxiliary pumps

#### Lube oil system with filters and Unity CC Jensen VRU oil cleaning skid

#### Main gearbox and starter/barring gear

#### Air filtration and cooling air skid

#### Exhaust stack and silencer assembly

#### HVAC ventilation for control shelter and generator house

#### Fire protection and CO₂ suppression system

#### Gas and fire detection, NOₓ and CO₂ monitoring skid

## BALANCE OF PLANT EǪUIPMENT

#### Fuel skid (gas and liquid) with separators, heaters, instrumentation

#### Starter transformer (10 kV/690 V dry type)

#### DC battery module and charger

#### MCC panels with UPS

#### Embedded foundation parts and installation spares

# INTERFACES

## MECHANICAL INTERFACES

#### Gas inlet connection flange

#### Exhaust flue tie-in to customer stack or HRSG

#### Cooling water supply and return

#### Lube oil and drainage connections

## ELECTRICAL INTERFACES

#### Generator HV terminal connection

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

Control power and auxiliary power terminations

SIEMENS SGT-600 PROPOSAL

#### Synchronization and protection wiring to customer switchboard

## INSTRUMENTATION G CONTROL INTERFACES

#### Signal terminations to customer DCS

#### Local I/O junction boxes for field devices

# TECHNICAL PERFORMANCES

#### ISO Performance Data (Simple Cycle):

#### Power Output: 25 MW

#### Electrical Efficiency: ~ 35%

#### Exhaust Temperature: ~ 570°C

#### NOx Emissions: <=15 ppmv (15% O₂)

#### Dual-fuel capability: Natural gas / diesel

# PROJECT EXPERIENCE

#### Over 300 SGT-600 units in operation worldwide with more than 4 million operating hours

#### Proven flexibility in distributed power, peaking, and emergency applications

#### Extensive service network in Europe, Asia, and the Americas

# OVERHAUL SCOPE

#### VG Engineering Consultants provides a complete overhaul and reinstallation scope for both Siemens SGT-600 units.

#### This includes, but is not limited to, the following:

### Gas Turbine Overhaul (Both Units A G B)

#### Level D inspection of the gas turbines

#### Disassembly, inspection, and reconditioning of major components

#### Reassembly and functional testing

### Gearbox Overhaul

#### Basic inspection of each main gearbox

#### Replacement of worn or out-of-spec parts as needed

#### Re-lubrication and operational testing

### Generator Overhaul

#### Basic inspection of both ABB LBA1120 LC generators

#### Testing of insulation and excitation systems

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

Replacement of serviceable parts if required

SIEMENS SGT-600 PROPOSAL

### Logistics and Handling

#### Removal of each unit from site and shipment to port (port Location)

#### Management of transport and documentation

#### Reinstallation on-site after overhaul completion

#### Not included to the OH scope:

### Installation G Commissioning

#### Site civil works (concrete, foundations, piping, etc.)

#### Reinstallation of overhauled equipment

#### Electrical and instrumentation loop-checks

#### Calibration and cold/hot commissioning support

# COMMERCIAL TERMS

Equipment: Two (2) complete SGT-600 Dual Fuel Turbine Generator Packages in OH condition Price: $17,250,000 USD (Seventeen Million Two Hundred Fifty Dollars) Each.

Delivery Terms: EXW Factory Europe. Shipping port to be confirmed.

Delivery Time: X months after order confirmation

#### Warranty: Twelve months or 2 000 operating hours (whichever occurs first) following successful overhaul, covering parts and workmanship.

### Payment Terms:

#### For standard equipment purchase:

#### 40% deposit upon contract signature

#### 40% upon completion of dismantling

#### 20% upon shipment from the factory For overhaul scope:

#### 50% upfront payment upon order confirmation

#### 50% upon completion of overhaul and readiness for dispatch

# CONCLUSIONS

#### The Siemens SGT-600 open cycle package offers a compact, reliable, and flexible solution for 25 MW power requirements.

#### VG Engineering Consultants is ready to support supply, installation, and commissioning to ensure optimal performance and uptime for your facility.

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

We appreciate the opportunity to submit this proposal and remain at your disposal for any further clarifications.

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL

VG ENGINEERING CONSULTANTS LTD

Registered in England and Wales under number: 0866 7924 | VAT registered in the UK under number: 170741126 Registered address: Chusan, Newtown Road, Penperlleni, Pontypool, NP4 0AW

www.vg-eng.com

SIEMENS SGT-600 PROPOSAL